AisleBot

Complete Setup & Deployment Manual

Asymmetric Mecanum Omnidirectional Robot for Narrow-Aisle Warehouse Navigation

Aritra Das  |  Roll No. 25D0074

Department of Biosciences and Bioengineering

Indian Institute of Technology Bombay

Supervisor: Prof. Ambarish Kunwar

GitHub Repository

https://github.com/AritraD11/Aislebot

# 1.  Fresh Pi — One-Click Install

On a brand-new Raspberry Pi 5 with Ubuntu 24.04 LTS installed, open a terminal and run this single command:

ONE-CLICK INSTALL COMMAND

bash <(curl -sSL https://raw.githubusercontent.com/AritraD11/Aislebot/main/install.sh)

This command downloads and runs the install script directly from GitHub. It will:

- Install ROS2 Jazzy Desktop (~15 min)

- Install Nav2, SLAM Toolbox, robot_localization, RPLiDAR, joy, rviz2

- Install Python packages: fastapi, uvicorn, pyserial, RPLCD, pandas, numpy, openpyxl

- Clone the AisleBot GitHub repository

- Build the ROS2 workspace with colcon

- Install udev rules (/dev/esp32 and /dev/mega)

- Install and configure the systemd autostart service

- Set up .bashrc with ROS2 environment and aliases

⚠ NOTE: Total time: approximately 20-30 minutes on a fresh Pi 5. Keep the Pi connected to the internet throughout.

# 2.  Prerequisites

## 2.1  Hardware Required

| Component | Detail |
| --- | --- |
| Raspberry Pi 5 | 4GB or 8GB RAM recommended |
| MicroSD Card | 32GB minimum, 64GB recommended (Class 10 / A2) |
| Power Supply | USB-C 5V 5A (official Pi 5 supply recommended) |
| Internet | Required during install — college WiFi or hotspot |
| Monitor + Keyboard | Only needed for initial Ubuntu setup and install |

## 2.2  Software Prerequisites

Before running the install command, the Pi must have:

- Ubuntu 24.04 LTS (Noble) installed on the SD card

- Internet connection active

- A user account (default username: aritra)

⚠ NOTE: Do NOT install ROS2 manually before running the script. The script handles everything.

## 2.3  Ubuntu 24.04 Flash Instructions

On your Windows laptop:

- Download Raspberry Pi Imager from raspberrypi.com/software

- Insert SD card into laptop

- Open Imager → Choose OS → Other general-purpose OS → Ubuntu → Ubuntu 24.04 LTS (64-bit)

- Choose Storage → your SD card

- Click the gear icon → set hostname: aritra-desktop, username: aritra, enable SSH

- Write → insert SD into Pi → boot

# 3.  What the Install Script Does (Step by Step)

The script runs 7 steps with progress output. Here is what each step does:

## Step 1  —  System update & base tools

Updates apt, installs git, curl, python3-pip, build-essential, i2c-tools, zip.

## Step 2  —  ROS2 Jazzy (~15 minutes)

Adds the ROS2 apt repository, installs ros-jazzy-desktop, colcon, and all required add-on packages including Nav2, SLAM Toolbox, joy, RPLiDAR, CycloneDDS, and rviz2.

## Step 3  —  Python packages

Installs exact versions matching the working Pi:

- fastapi 0.136.1  —  phone dashboard web server

- uvicorn 0.46.0   —  ASGI server for FastAPI

- pyserial 3.5     —  serial communication with ESP32 and Mega

- RPLCD 1.4.0      —  16x2 I2C LCD display driver

- pandas 3.0.0     —  telemetry data analysis

- numpy + openpyxl —  data processing and Excel export

## Step 4  —  Clone AisleBot repo

Clones https://github.com/AritraD11/Aislebot into /tmp, then copies the src/ packages into ~/ros2_ws/src/.

## Step 5  —  Build ROS2 workspace

Runs colcon build --symlink-install in ~/ros2_ws. All 9 nodes are compiled and installed.

## Step 6  —  System configuration

- Installs /etc/udev/rules.d/99-aislebot.rules — gives ESP32 and Mega persistent device names

- Installs ~/start_aislebot.sh — the launch script called by systemd

- Installs /etc/systemd/system/aislebot.service — autostart on boot (optional)

- Adds user to dialout group (serial port access)

- Adds user to i2c group (LCD access)

## Step 7  —  Shell environment

Adds to ~/.bashrc:

- source /opt/ros/jazzy/setup.bash

- source ~/ros2_ws/install/setup.bash

- export ROS_DOMAIN_ID=42

- export RMW_IMPLEMENTATION=rmw_cyclonedds_cpp

- CycloneDDS loopback URI (required for headless Pi with no LAN)

- Aliases: ab, ab-build, ab-log, ab-status, ab-start, ab-stop, ab-ports

# 4.  After Install — First Boot Steps

## 4.1  Plug in hardware (order matters)

⚠ NOTE: Always plug ESP32 (CP2102) FIRST, then Arduino Mega (CH340). The udev rules assign /dev/esp32 and /dev/mega based on USB vendor IDs, so order does not strictly matter — but this is the safe habit.

Verify devices are detected:

ls /dev/esp32 /dev/mega

# Expected output:

# /dev/esp32  /dev/mega

## 4.2  Flash firmware (from Windows laptop)

| Component | Detail |
| --- | --- |
| ESP32 firmware | aislebot_esp32_v2.ino → ESP32-WROOM-32 @ 921600 baud |
| Arm firmware | aislebot_arm_v7.ino → Arduino Mega 2560 @ 115200 baud |
| Arduino IDE | Tools → Board → ESP32 Dev Module (CPU: 240 MHz) |
| Partition | Default 4MB with spiffs |

## 4.3  Open a new terminal

After install, open a new terminal (or run source ~/.bashrc) so the ROS2 environment is loaded.

## 4.4  Launch the robot

ab

# Same as:

# ros2 launch mecanum_robot aislebot_full.launch.py

Expected startup sequence:

- ESP32 bridge connects on /dev/esp32

- Arduino arm bridge connects on /dev/mega — responds PONG, enables arm

- LCD shows AISLEBOT READY + IP address

- Phone dashboard ready at http://<PI_IP>:8080

- Xbox controller detected (if plugged in)

## 4.5  Open phone dashboard

On your phone, connect to the same WiFi network as the Pi, then open:

http://10.53.6.122:8080

# Find your Pi's IP with:

hostname -I

# 5.  Hardware Reference

## 5.1  Full hardware stack

| Component | Detail |
| --- | --- |
| Compute | Raspberry Pi 5, Ubuntu 24.04.4 LTS, ROS2 Jazzy |
| Drive controller | ESP32-WROOM-32 38-pin CP2102 → /dev/esp32 @ 921600 baud |
| Arm controller | Arduino Mega 2560 CH340 → /dev/mega @ 115200 baud |
| Motors | Rhino RMCS-2086 (24V, 60 RPM, 1:47 gear, 93132 CPR) |
| Motor drivers | 2× Cytron MDD20A |
| Wheels | DekuPro 6-inch SR Mecanum (radius = 0.0762 m) |
| Power | SM12830SL LiFePO4 battery + SSR-50DD solid state relay |
| UV arm | 2× NEMA23 (TB6600) + NEMA34 linear actuator (BH-MSD-6A-W) |
| LCD | 16×2 I2C (PCF8574 at 0x27, SDA/SCL) |
| LiDAR | RPLiDAR A1 (planned) |
| IMU | BNO055 (planned) |
| Level shifter | 2× TXS0108E (5V→3.3V for encoder signals) |

## 5.2  Robot geometry (asymmetric)

| Component | Detail |
| --- | --- |
| Outer wheel pair (FR, RL) | l1 = 0.403 m from centre |
| Inner wheel pair (FL, RR) | l2 = 0.333 m from centre |
| Half-track width | d = 0.15769 m |
| K_outer = l1 + d | 0.5607 m (used in IK for FR and RL) |
| K_inner = l2 + d | 0.4907 m (used in IK for FL and RR) |
| Wheel radius | 0.0762 m (DekuPro 6-inch: 152.4mm OD / 2) |

⚠ NOTE: K_outer and K_inner MUST be identical in: ESP32 firmware, mecanum_teleop_asymmetric.py, odometry_publisher.py, and nav2_params.yaml. Any mismatch causes drift and incorrect odometry.

# 6.  ROS2 Node Architecture

## 6.1  All 8 nodes

| Component | Detail |
| --- | --- |
| esp32_bridge | Serial ↔ ROS2 bridge for ESP32. Sends wheel velocity commands, receives telemetry. |
| teleop_asym | Converts /cmd_vel Twist → /wheel_speeds using asymmetric mecanum IK. |
| odom_pub | Reads encoder feedback → publishes /odom (odometry/filtered). |
| arm_bridge | Serial ↔ ROS2 bridge for Arduino Mega arm controller. |
| joy_to_aislebot | Converts /joy (Xbox) → /cmd_vel and /arm/command. |
| phone_dashboard | FastAPI + WebSocket HTTP server on port 8080. Phone control interface. |
| lcd_display | Writes status to 16×2 I2C LCD at address 0x27. |
| joy_node | ROS2 joy package — reads USB Xbox controller → /joy topic. |

## 6.2  Topic map

| Component | Detail |
| --- | --- |
| /joy | sensor_msgs/Joy — Xbox controller raw input |
| /cmd_vel | geometry_msgs/Twist — drive commands (vx, vy, wz) |
| /wheel_speeds | std_msgs/Float64MultiArray — [FR, FL, RR, RL] rad/s |
| /arm/command | std_msgs/String — arm commands (LIFT, LOWER, OPEN, CLOSE, ESTOP) |
| /odom | nav_msgs/Odometry — wheel encoder odometry |
| /motor_telemetry | std_msgs/Float64MultiArray — [FR_t, FR_a, FR_pwm, FL_t...] 12 values |
| /esp32/command | std_msgs/String — raw commands to ESP32 (<L1>, <S>, <E1>, etc.) |

## 6.3  Useful aliases (added to .bashrc by install script)

| Component | Detail |
| --- | --- |
| ab | ros2 launch mecanum_robot aislebot_full.launch.py |
| ab-build | colcon build --symlink-install in ~/ros2_ws |
| ab-log | tail -f ~/aislebot_boot.log |
| ab-status | sudo systemctl status aislebot |
| ab-start | sudo systemctl start aislebot |
| ab-stop | sudo systemctl stop aislebot |
| ab-ports | ls /dev/esp32 /dev/mega (check hardware connected) |

# 7.  Keeping the GitHub Repo Up to Date

After any code change on the Pi, sync it to GitHub so the install script always restores the latest version:

cd ~/aislebot

# Copy changed files from workspace to repo folder

cp ~/ros2_ws/src/mecanum_robot/mecanum_robot/*.py  src/mecanum_robot/mecanum_robot/

cp ~/ros2_ws/src/mecanum_robot/launch/*.py         src/mecanum_robot/launch/

# Commit and push

git add -A

git commit -m "update: describe what changed"

git push

⚠ NOTE: If you also changed system files (start_aislebot.sh, aislebot.service, udev rules), copy those to ~/aislebot/system/ before committing.

# 8.  Troubleshooting

## 8.1  Service fails at boot

Check the boot log:

cat ~/aislebot_boot.log

Common cause: ESP32 not plugged in. The service waits 30 seconds for /dev/esp32 then exits. Plug in ESP32 and run:

sudo systemctl start aislebot

## 8.2  /dev/esp32 or /dev/mega not found

lsusb  # check USB devices are detected

ls /dev/ttyUSB*  # fallback device names

sudo udevadm control --reload-rules && sudo udevadm trigger

## 8.3  LCD errors (Remote I/O error)

LCD is physically disconnected from the robot. The node logs warnings but continues running — this is not a fatal error.

## 8.4  Phone dashboard not loading

# Check dashboard is running

ros2 node list | grep phone

# Get Pi IP

hostname -I

# Make sure phone is on same WiFi as Pi

## 8.5  colcon build fails

cd ~/ros2_ws

source /opt/ros/jazzy/setup.bash

colcon build --symlink-install 2>&1 | grep -i error

## 8.6  CycloneDDS warning in logs

The warning 'NetworkInterfaceAddress: deprecated element' is harmless. It appears in all nodes but does not affect functionality. The loopback config works correctly.

# 9.  PID Tuning Reference

## 9.1  Current PID gains (ESP32 firmware)

| Component | Detail |
| --- | --- |
| Kp | 50.0 — proportional gain |
| Ki | 30.0 — integral gain |
| Kd | 3.0 — derivative gain |
| Loop rate | 50 Hz (every 20 ms) on Core 1 |
| Feedforward FR | 42.1 PWM per (rad/s) |
| Feedforward FL | 40.2 PWM per (rad/s) |
| Feedforward RR | 43.7 PWM per (rad/s) |
| Feedforward RL | 47.9 PWM per (rad/s) |

## 9.2  Live gain tuning via serial

Connect ESP32 to laptop via USB (921600 baud), open Serial Monitor:

<G,50,30,3>      # set Kp=50 Ki=30 Kd=3

<F,42.1,40.2,43.7,47.9>  # set feedforward gains

<L1>             # enable telemetry CSV output

<L0>             # disable telemetry

<V,2.0,2.0,2.0,2.0>  # spin all wheels at 2 rad/s

<S>              # E-STOP (latches)

<E1>             # clear E-STOP and re-enable

<?>              # live state of all motors

## 9.3  Telemetry CSV format

When <L1> is active, ESP32 outputs CSV at 10 Hz:

timestamp_ms, FR_target, FR_actual, FR_pwm, FL_target, FL_actual, FL_pwm, RR_target, RR_actual, RR_pwm, RL_target, RL_actual, RL_pwm

The Pi's phone_dashboard saves this to ~/aislebot_logs/run_YYYYMMDD_HHMMSS.csv when Record Run is pressed.

## 9.4  Known motor performance (air test baseline)

| Component | Detail |
| --- | --- |
| FL (Front Left) | 2.983 rad/s, CV 0.33% — fastest, most consistent |
| FR (Front Right) | 2.853 rad/s — solid |
| RR (Rear Right) | 2.743 rad/s — slightly weaker |
| RL (Rear Left) | 2.507 rad/s, CV 4.57% — slowest, most erratic |
| FR↔RL diagonal gap | 11-13% — primary source of stutter. PID addresses this. |

# 10.  Quick Reference Card

FRESH PI RESTORE — ONE COMMAND

bash <(curl -sSL https://raw.githubusercontent.com/AritraD11/Aislebot/main/install.sh)

DAILY LAUNCH

ab

# or: ros2 launch mecanum_robot aislebot_full.launch.py

CHECK HARDWARE

ab-ports # /dev/esp32 and /dev/mega

ab-log # live boot log

ab-status # systemd service status

UPDATE GITHUB AFTER CODE CHANGE

cd ~/aislebot

cp ~/ros2_ws/src/mecanum_robot/mecanum_robot/*.py src/mecanum_robot/mecanum_robot/

git add -A && git commit -m 'update' && git push

GITHUB REPO

https://github.com/AritraD11/Aislebot

AisleBot — IIT Bombay — Aritra Das (25D0074)